ASEN 2002 Balloon Experiment (Example Schedule)

Educational Goals:

- Buoyancy Force, neutral buoyancy, force balance

- Theoretical vs. Empirical

- Experimental methods

- Thermodynamic effects on buoyancy

- Thin Walled Pressure Vessels

Experimental Schedule

Demo:

- Discuss force balance

- Discuss thin walled pressure vessel equation

- Inflate balloon

- Add payload to neutral buoyancy

- Increase payload (so it doesn’t float away) and hand around

(Have students think about how to estimate volume)

- Tell them they will have to estimate radius of burst

- Inflate balloon to failure (~5g)

Lab Day 1:

- Assuming 55 students, implies around 11 groups

- 6 groups perform heating experiment

- 5 groups perform neutral buoyancy, and volume measurements

Lab Day 2:

- Switch group tasks from Lab Day 1

Experimental Procedure:

Legend:

- 1, 2, 3… = Steps

- Arrow-Pointer = Note for Instructors/Course Assistants

Part 1:

- Weigh balloon

- Fill balloon using tank attached to mass flow-meter

  - CA/Students documents mass estimate

- Empirically add payload until neutral buoyancy is achieved

- Remove and weigh payload (total) mass

  - Students should note any zip-ties, strings, etc (not negligible masses)

- Measure water height in volume tank

- Submerge balloon, let water settle, get Δh and calculate Volume

  - Students must account for their hand’s volume

  - Students should get better dimensions for L and W

- Estimate mass of gas

Part 2:

- Fill balloon

- Add payload to slightly negative buoyancy

  - Must be slight

- Place balloon in front of heater

- Watch balloon rise and fall

- Document maximum height achieved (approximately)

- Detach payload and weigh

- Calculate required Δ Volume to lift payload

- Estimate Δ Temperature to achieve necessary Δ Volume (ideal gas law)

- Relate Δ Volume to height or acceleration

Questions for the Students:

- How would you estimate the volume of this shape?  What volumes do you achieve? Are they reasonable?

  - Should be answered prior to experiment, or at least displayed that estimates were made

- At what radius do you estimate the balloon will rupture? Is that reasonable?

  - Students should calculate and compare to the estimate from the lab demo

- If you were to make your balloon neutrally buoyant in water, what payload mass would be required?

  - Should be ~1000 times their aerial mass

- Would a different gas make a significant difference for buoyancy in air? In water?

- How would you improve the experiment / measurements?

- If you desired to lift more mass, what could you change? What is its effect?